Example on Unit II: Logical Link Control                                 Computer Networks TY(Comp)

Prepared By: M L Dhore

Network Performance Measured by: Bandwidth (Throughput) and Latency (Delay). Book: Computer Networks by L L Peterson and B S Davie

BW :

- No. of bits transmitted over the network in a certain period of time.

- How long it takes to transmit a data?

- 10 Mbps means 10 million bits delivered per second.

- Mbps means 106  Or  1 000 000

- 1/10 Mbps = 0.1 microsecond (µs) to transmit each one bit.

- Microsecond means  10-6

- It means one bit is 1 µs wide over the medium in terms of time.

- For 2 Mbps, 1/2Mbps = 0.5 µs wide

Latency:

- How long it takes to travel from one end to another end of network?

- Measured in terms of time

- 24 milliseconds means it takes 24 ms to deliver the message from one end to another

- RTT Round Trip Time is the two way time taken.

Latency = Propagation Time + Transmit Time + Queuing Delay

Propagation Time is the propagation delay wrt the speed of light latency

For vacuums = 3 x 108 m/s ie for unguided media

For cable = 2.3 x 108 m/s ie for guided media

For Fiber = 2.0 x 108  m/s

Propagation Time = Distance / Speed of Light

Propagation Time = Distance of wire / Effective Speed of Light over wire

PT = d/v   where d is the distance and v is velocity of propagation

Transmit Time: It is the amount of time required to transmit the given amount of data.

Transmit Time = Transfer Size / Bandwidth   Note: Transfer Size is the amount of Data

Transmit Time = Length of frame /data transfer rate

TT = L /R

Delay x Bandwidth Product

delay

BW

Consider latency corresponds to the length of pipe and diameter of pipe is the bandwidth then product of length and BW gives the volume of pipe ie no. of bytes that could be transit at any given instant.

Throughput:

Throughput = Transfer Size / Transfer Time

where     Transfer Time = PT + Transmit Time

= RTT + (Size of Data / BW)

= RTT + (1 / BW)  x   Transfer Size

Performance of Stop and Wait

Time to send data TD = TI + n TF   where TD  is Propagation Delay

where TI = Time for initialization, TF = Time to send one frame, n = no. of frames

TD = n( 2 tprop time + tframe)

Utilization u = tframe /2 tprop time + tframe

u = TT/TT = Transmit Time/Transfer Time

Let   a =  tprop time / tframe

If a<1, then   tprop time < tframe

If a>1, then   tprop time > tframe

Then

Utilization u = 1/ 1 + 2a

We know

tprop time  = distance / velocity = d/v

tframe =  length of frame (Size) / data rate (BW)  =  L /R

Therefore   a = tprop time / tframe =  (d/v) / (L/R) = Rd/vL

Throughput = L / ( tframe + RTT)

Channel Efficiency = (L/R) /(RTT + L/R) or  Throughput in bps / DTR

Example of Calculating Propagation Time

Ex.1:  Find the RTT (Round Trip Time) for the dial up connection over telephone cable for distance 10 km

Solution:

Example of Calculating Latency

Ex.2:  Find the RTT (Round Trip Time) for the dial up connection over telephone cable for distance 10 km. Calculate the latency to transfer 1 MB data with 1 Mbps bandwidth.

Solution:

Latency = PT + TT + Queuing Time

Consider queuing time is negligible then

Latency = PT + TT =

Latency Vs Bandwidth

Case I: For small amount of data latency dominates bandwidth.

Ex.3  Find the transfer time for 1 byte of data over 1 Mbps and 100 Mbps bandwidth with 1 ms RTT and 100ms RTT.

Solution:

Case II: For large amount of data bandwidth dominates latency

Ex.4  Find the transfer time to transfer 25 MB of data over 1 Mbps and 100 Mbps bandwidth with 1 ms RTT and 100ms RTT

Solution:

Delay x Bandwidth Product

delay

BW

Consider latency corresponds to the length of pipe and diameter of pipe is the bandwidth then product of length and BW gives the volume of pipe ie no. of bytes that could be transit at any given instant.

Ex.5 Calculate the delay x bandwidth product for one way latency equal to 50 ms and bandwidth is 45 Mbps.

Solution:

| Link Type | Velocity | BW | Distance | RTT | Delay x BW |
| --- | --- | --- | --- | --- | --- |
| Dial-up | 2.3 x 108 m/s | 56 Kbps | 10 Km | 87 µs | 5 bits |
| Wireless LAN | 3 x 108 m/s | 54 Mbps | 50 m | 0.33 µs | 18 bits |
| Satellite | 3 x 108 m/s | 45 Mbps | 35,000 km | 230 ms | 10 Mb |
| Cross country Fiber | 2 x 108 m/s | 10 Gbps | 4000 km | 40 ms | 400 Mb |

Ex.6 Calculate the delay x bandwidth product considering the RTT for v= 2.3 x 108 m/s, R= 56Kbps and distance= 10Km. Find the 1-bit length in time and meters or Kilometers?

Solution:

-------------------------------------------------------------------------------------

Ex.7 Calculate the delay x bandwidth product considering the RTT for v= 3 x 108m/s, R= 54 Mbps and distance= 50m. Find the 1-bit length in time and  meters or Kilometers?

Solution:

-------------------------------------------------------------------------------------

Ex. 8 Find the throughput to transfer 1 MB file over 1 Mbps bandwidth and 1Gbps network. RTT is 100 ms for both networks.

Solution:

Case I: 1 MB file over 1 Mbps line

Transfer Time = RTT + (Transfer Size /BW)

Transfer Time/ BW= 1 MB / 1 Mbps

= 1 000 000 x 8 / 1 000 000 s

= 8 000 000 / 1 000 000 s

= 8 sec

= 8000 ms

Therefore to transfer a file of 1 Mb over 1 Mbps line with RTT 100 ms, it takes 80 RTT ie 80 x 100 ms = 8000 ms.

Transfer Time = 100 ms + 8000ms = 8100 ms

Throughput = Transfer Size / Transfer Time

Throughput = 1 MB / 8100 ms

= 1 x 106 x 8 / 8100 x 10-3 s

= 8 x 106 / 8.100 s

= 0.987 Mbps

= 987 kbps

Case II: 1 MB file over 1 Gbps line

Transfer Time = RTT + (Transfer Size /BW)

Transfer Time/ BW= 1 MB / 1 Gbps

Transfer Time = 1 MB / 1 Gbps

= 1 000 000 x 8 / 1 000 000 000 s

= 8 000 000 / 1 000 000 000 s

= 8 ms

Therefore to transfer a file of 1 Mb over 1 Gbps line with RTT 100 ms, it does not take even a 1  RTT

Transfer Time = 100 ms + 8ms = 108 ms

Throughput =   1 MB / 108 ms

= 1 x 106 x 8 / 108 x 10-3 s

= 8 x 106 / 0.108 s

= 74.07 Mbps

Examples 9,10,11 from:Computer Networks by L L Peterson and B S Davie

Ex 9: Calculate the total time required to transfer a 1.5 MB file in the following cases, assuming a RTT of 80 ms, a packet size of 1 KB data and initial 2xRTT of handshaking before data is sent.

- The bandwidth is 10 Mbps, and data packets can be sent continuously.

- The bandwidth is 10 Mbps, but after we finish sending each data packet, we must wait one RTT before sending the next.

- The link allows infinitely fast transmit, but limits BW such that only 20 packets can be sent per RTT.

-------------------------------------------------------------------------------------

Ex 10: Consider point-to-point link 50 km in length. At what BW would propagation delay (at a speed of 2 x 108 m/s) equal transmit delay for 100-byte packets? What about 512-byte packets?